9/2018

Specificationsmaychangewithoutnotice.

Petrochemical

Permanent Gases & Natural Gas

www.dps-instruments.com

The DPS Perma-gas 1 GC System is ideal for separating the whole

gas components Hydrogen, Oxygen, Nitrogen, Methane, Carbon

Monoxide and Carbon Dioxide with one injection. Additionally, C2

through C6 hydrocarbons are easily separated in the same analysis.

The sensitive and universal Helium Ionization Detector (HID) from

DPS and our innovative 2 column and valve configuration simplifies

this analysis. The DPS Perma-gas 1 GC Systems are ideal for ppm

level measurements in your high percentage gas samples. Perma-gas

1 GC Systems can be built into our Series 600 Lab GC, or the

Portable Companion 2, allowing you to take the analyzer with you into

the field. Only a small tank of Helium is need to operate the GC

System. The fast heating and rapid cooling column oven in every DPS

GC assures quick sample turnaround. The fully integrated Perma-gas

1 GC Analyzer Systems are small and lightweight and all DPS

systems are modular for expandability, upgrades, and easy service.

Available Configurations Include:

600-C-075 - Series 600 Perma-Gas 1 GC Analyzer (HID, Valve, 2 Columns)

500-C2-075 - Companion 2 Portable Perma-Gas 1 GC Analyzer (HID,

Valve, 2 Columns)

Series 600 GC

Permanent Gas Standard - 1000 ppm

Component	Area  ppm

Hydrogen	831.2  1000

Oxygen	2722.6  1000

Nitrogen	2147.6  1000

Methane	7037.0  1000

CO	3685.2  1000

Ethane	24484.2  1000

CO2	7996.0  1000

Ethylene   19515.4  1000

Propane	30906.7  1000

Acetylene  18363.6  1000

Propylene  27521.3  1000

HID Detector

Detector Temperature = 200C

Collector = -100V

High Voltage = 800V

Carrier = Helium @ 200 kPa

Column = Mol Sieve & Silica Gel in Series

Temp Program = 60C (4 min) to 220 @ 10C/min

Companion 2 Portable GC

DPS Companion 2 Perma-Gas GC Layout

Valve Oven

Small High Pressure

Gas Cylinders

Rugged

watertight

case

Gas Connections

GC Oven

Power connection

with breaker

and line filter

HID Detector

HID Detector

Color Touchscreen

USB & Ethernet

Connections

Heated On-Column Injectors

Plumbing Diagram

Sample Analysis - The Gas Sample Valve and heated Valve Oven for the Companion GC’s are built

right in to provide the shortest possible sample path. The Sample Line is connected to the Valve

Oven and from there all of there the entire

sample path is heated to limit possible carry

over. A fixed Sample Loop ensures

reproducible sampling and is Flushed between

analyses. The sampling and analysis

sequence is automated through the Timeline

of the DPS GC Control Software. The analysis

can be set up to run unattended 24/7

collecting, processing, and storing all of the

data.

The unique 2 column configuration simplifies

the compound separation and analysis. The

columns are plumbed in series through the

heated Sample Valve.

Plumbing Diagram - In the 1st Step the

sample is loaded on the Sample Loop with the

built-in vacuum pump. During Step 2 the

Sample Valve is rotated to Inject the sample

onto the analytical columns. The Silica Gel

column retains CO2 & the C2+ hydrocarbons,

while the lighter compounds (H2, O2, N2,

CH4, & CO) pass through and are further

separated on the Molecular Sieve column.

Once the lighter compounds have been

separated the valve is rotated back in Step 3

and the heavier compounds (CO2 & C2+

hydrocarbons) are separated on the Silica Gel

column.

Results, Data & Connectivity

Perma-Gas One

Plumbing Diagram

Results: The results and chromatogram are stored on the hard drive. Additionally, for each

channel a log file summary of the compounds detected is a convenient way of looking at large

amounts of data collected over time.

Data and Connectivity: The built-in computer is used to collect and store the data. Data can

also be copied to a USB Stick to transfer to another computer. Data can be transferred from the

built-in computer to another computer on the LAN through the Ethernet port using standard

Windows protocols. Or, we can use a USB cable to connect the GC to the remote computer where

the data can be collected and stored on that hard drive.

Plumbing Connection Summary

Sample Point - The vacuum pump is strong enough to transfer the sample

from the source to the GC as far away as 20 meters using a flexible and

inert tube.

Calibration Gas - The pressurized gas flowing through a transfer tube can be

100 meters, or more from the GC. A solenoid inside the GC opens to let the

calibration gas flow through the Sample Loop. When the solenoid closes the

gas equilibrates to ambient pressure before injection.

Carrier Gas - The small high pressure tank fits right inside the GC. Or a large

cylinder can be connected for a longer lasting carrier gas supply. There is no

limit on the distance from the cylinder to the GC.

Sample Point

Calibration

Gas

Large Gas

Cylinder

DPS Companion Accessories

Gas Sample Valve

Sample Inlet	Sample Valve

Sample Loop

Heated Lines

Small High Pressure Refill Kit

ON/OFF Valve

Small Tank

Pressure Gauge

High Pressure

Cylinder

SLP Regulator

Fill Pressure Gauge

Connection to

Main Tank

Fill/Pressure

Release Knob

Tank Pressure

Gauge

Accessory Kits

GC Maintenance Kit

Tools, Keyboard, Mouse,

Voltmeter

Gas Line Kit

Regulator, Tubing,

Cutters, Fittings

Shipping Kit

Syringes, Power Cord,

Nuts, Ferrules, Screws

(Included with each GC)

DPS GC Control Software

Easy to learn and master using a

Graphical User Interface (GUI) and

Color Touch Screen.

Editors let you customize the files

associated with the GC Method.

Oven Temp Program Editor

Method Name

Save the current

name or create a

new one

GC Program Page

Timeline Editor

File Selection Arrows

Navigation Buttons to Quickly jump

from one screen to another.

Most pages are one button away!

Carrier Pressure 1 Editor

Diagnostics

Keyboard

to Enter Filenames

Number Pad

for entering Values

Carrier Pressure 2 Editor

GC Status pages display the

parameters in the method, both

graphically and as text and values.

Oven Status

Method Editor

System Status

Detector Status

Run Status

System status

pages display

the health

and viability

of the GC

instrument.

Hardware	Configure Names

Diagnostics

Plumbing            Remote Control

- 400  C Temperature Limit with 0.1  C set-point resolution

Perma-Gas GC Specifications:

Electronics Module:

- Enter and store GC Methods via Color Touch Screen

- Actual and set-point display of all GC parameters

- Safety Limits on all user entered parameters

- Oven Temperature Programs (OTP) with Multiple Ramps

- Pressure Programs for Carrier Gases with Multiple Ramps

- Timeline for sequencing Relays and Valve

- Detector Control of all Parameters on one page

- Electronic Pressure Controllers (EPC’s):

Atmospheric Pressure & Temperature Compensation

EPC Pressure Control with 0.1 kPa set-point resolution

- Plug and Play GC Control, Oven, and Detector Board

- Microprocessor Controlled

- Proprietary Digital Signal Processing

- Digital Signal Outputs for each Detector

- Universal voltage input (85 – 240 Vac) with line

filter and breaker.

Detector:

HID – Helium Ionization Detector (10 ppm detection limit,

dependent on sample loop size)

o	o

- 24-bit Digital Outputs for the detector via USB

- EPC Pressure Control with 0.1 kPa set-point resolution

Columns:

1m Molecular Sieve

2m Silica Gel

Results:

Automatically calibration corrected and reported in % or ppm

Lab Quality Analyses in the Field,

“It Goes with you Anywhere!”

Series 600 Oven Module:

- Ambient to 400oC Column Oven

- Up to 100 oC per/min Oven Ramp

- Fast Cooldown 300 oC to 50 oC in 3.5 min

- 1000 watt total Heater Elements

- Temperature Ramps with 0.1 oC set-point resolution

- 23 x 23 x 20 cm area for Glass, SS, or Capillary Columns

Companion 2 Oven Module:

- Ambient to 325 oC Column Oven

- Up to 80 oC per/min Oven Ramp

- Fast Cooldown 300 oC to 50 oC < 4 min

- 200 watt Heater Element

- Temperature Ramps with 0.1 oC set-point resolution

- 12.5 x 10.5 x 12.5 cm area for Packed, or Capillary Columns

FCO Oven Module:

- Sub-Ambient to 450 oC “Fluidless” Column Oven

Built-In Accessories:

- Sample Valve - Electronically Actuated

- Heated Valve Oven

Injector:

- Heated On-column Injector

- Multiple Pressure Ramps with 0.1 kPa set-point resolution

Data Communications:

- Bi-directional communication with popular Data System

Network Connectivity:

- Enterprise Compatible Network GC running Windows XPe

- Ethernet Connection using Windows Network Protocol

- On Board ETX Computer for GC Control and

Data Acquisition

- Remote Control of GC and Data Acquisition

over LAN